# Summary: Using Quantum Technology to Save Energy Costs in Factories

Paper 1 link and Summary

The Problem They Want to Solve

Electricity prices keep rising, which makes manufacturing more expensive. To stay competitive, factories need to lower their energy bills. One way to do this is by using their own renewable energy (like solar panels), buying electricity from the grid when it’s cheap, and storing extra energy in batteries for later use. However, this is complicated because:

- Solar power depends on weather (unpredictable).

- Electricity prices change throughout the day.

- Factory energy needs also change constantly.

So, factories need a smart system that can decide, in real time, when to use grid power, when to use solar power, and when to charge or discharge batteries—all to minimize costs. This is a difficult optimization problem, especially when conditions change suddenly.

The Technology They Used

Instead of using traditional computers, the researchers used a new technology called Quantum Annealing. This is a type of quantum computing that is very good at finding the best possible answer among many options in a very short time. They formulated the factory’s energy decisions as a mathematical problem and solved it using a special quantum solver from a company called D-Wave.

How They Tested It

They created two simplified test scenarios (use cases) for an 8-hour factory shift:

- Use Case 1: They tested how well the system scales by dividing the shift into different time intervals (15, 60, and 120 minutes). They wanted to see if the computation time would explode as the problem got more complex.

- Use Case 2: They simulated a sudden change mid-shift (less renewable energy became available) to see if the system could quickly recalculate a new plan.

Key Results

- The quantum system found optimized energy plans within seconds for all test cases.

- Even when the problem became more complex (smaller time intervals meant more decisions to make), the computing time increased only slightly—showing excellent scalability.

- When conditions changed (Use Case 2), the system rescheduled a new optimal plan in just a few seconds, demonstrating great flexibility---

Why This Matters

This research shows that quantum annealing can be used to build a fast, practical energy management system for factories. It can respond to changing weather, price fluctuations, or production needs almost instantly, helping manufacturers save money and use energy more efficiently. The approach is promising because it solves in seconds what traditional methods might take much longer to compute, especially for larger factories

Next Steps for Research

The authors plan to compare their quantum approach with traditional algorithms to confirm its superiority. They also want to include more real-world factors like peak load penalties, equipment depreciation costs, and multi-objective optimization (e.g., balancing cost with environmental impact).

# Paper Summary: Federated and Quantum-Inspired AI in Adaptive Traffic Systems

Lint to Paper 2

Problem Statement

Urban traffic congestion in Indian megacities (e.g., Delhi) causes massive economic losses (~₹1.5 lakh crore/year) and environmental damage. Traditional traffic control systems (fixed timers, SCATS, SCOOT) fail under chaotic mixed traffic (two-wheelers, EVs, trucks, pedestrians) and cannot handle real-time disruptions like accidents, monsoons, or festivals. Additionally, privacy laws (India's DPDP Act, GDPR) prohibit centralizing camera footage and vehicle trajectories, forcing operators to work in data silos.

Proposed Solution: FedQ-Twin Framework

The authors propose an integrated platform combining four technologies into one end-to-end system:

| Technology | Role |
| --- | --- |
| Federated Learning | Edge nodes (intersections) train local AI models on their own camera/V2X data. Only model gradients are shared — no raw video or license plates are transmitted, ensuring GDPR/DPDP compliance. |
| Quantum-Inspired Optimization | Signal timing is formulated as an Ising Hamiltonian (QUBO problem) and solved using simulated bifurcation machines on classical GPUs — exploring combinatorial spaces 5x faster than deep reinforcement learning. |
| Digital Twin | Virtual replicas of intersections built in SUMO + CARLA co-simulation, fused with real-time LiDAR/camera/radar via Unscented Kalman Filters. Enables safe "what-if" testing before real deployment. |
| LSTM-Transformer Hybrid | Predicts congestion 15-60 minutes ahead with 92% accuracy — LSTM captures time patterns, Transformer captures spatial dependencies across intersections. |
| COPERT Emission Model | Estimates CO₂ per vehicle based on speed, acceleration, and fuel type — emissions are embedded directly into the optimization objective (not an afterthought). |

System Architecture

- Sensing Layer: Roadside units (RSUs) capture vehicle counts, speeds, and platoons via YOLOv8 on 4K cameras, fused with inductive loops and V2X beacons over 6G MQTT streams.

- Federated Core: Each intersection trains a GNN-LSTM hybrid locally. FedAvg aggregates gradients at a lightweight cloud coordinator — feeding delay-emission forecasts into the optimizer.

- Quantum-Inspired Optimizer: Maps forecasts onto an Ising Hamiltonian where spins encode green/red phases. Solves signal timing 5x faster than deep RL.

- Digital Twin Validation: Simulates interventions with 95% fidelity before physical rollout — testing accidents, flash crowds, and special events.

- Deployment: ONNX-portable microservices run on Kubernetes-orchestrated edge clusters (Raspberry Pi to NVIDIA Jetson) with sub-200ms latency.

Experimental Results

Tested on Delhi-inspired grid in SUMO/CARLA with 10,000+ vehicle agents calibrated to India's mixed fleets.

| Metric | Fixed-Time Control | FedQ-Twin (Proposed) | Improvement |
| --- | --- | --- | --- |
| Average Delay per Vehicle | 95 s/veh | 68 s/veh | 28% reduction |
| Total CO₂ Emissions | 100 kg/h | 78 kg/h | 22% reduction |
| Mean Queue Length | 24 vehicles | 17 vehicles | 29% reduction |
| Control Latency | 320 ms | < 200 ms | 38% faster |
| Prediction Accuracy | — | 92% (60-min ahead) | — |
| Convergence Speed | — | 5x faster than Deep RL | — |

Comparative Baselines

- Outperforms Fixed-Time Control, Deep RL (PPO), and GNN-based adaptive control in all metrics.

- Federated learning reduces communication overhead by 70% while maintaining accuracy.

Key Innovations

- Privacy-Preserving: Federated learning with differential privacy enables collaborative training without exposing sensitive vehicle/trajectory data.

- Emissions-Aware: COPERT emission model integrated directly into the optimization objective — not a post-hoc calculation.

- Closed-Loop Validation: Digital twin verifies quantum-optimized signal plans before deployment, eliminating real-world risks.

- Real-Time Ready: Sub-200ms latency and Kubernetes orchestration make it deployable on existing SCATS infrastructure.

- Explainability: SHAP-based XAI provides regulatory audit trails for city operators.

Limitations

- Simulation-Only: Validated in SUMO/CARLA — not yet tested in real-world deployment.

- Quantum-Inspired, Not Real Quantum: Uses classical GPUs simulating quantum annealing — no actual D-Wave or quantum hardware.

- Traffic-Only: Does not cover route guidance, public transit integration, or pedestrian flow.

- Edge Deployment Overhead: Real-world communication delays and sensor noise still need extensive testing.

# Paper Summary: Leveraging Quantum Annealing for Layout Optimization

Link to paper 3

Problem Statement

Wind Farm Layout Optimization (WFLO) is a critical challenge in renewable energy. The objective is to strategically position wind turbines to maximize energy production while minimizing wake losses — the reduction in wind speed caused by upstream turbines that can significantly degrade power output. This problem is nonlinear, combinatorial, and NP-hard, making it extremely difficult for traditional optimization methods. The authors investigate whether quantum computing (specifically quantum annealing) can solve this problem faster than classical state-of-the-art solvers.

Proposed Solution

The authors formulate WFLO as a Quadratic Unconstrained Binary Optimization (QUBO) problem — a format compatible with quantum annealers. They compare three optimization approaches:

| Approach | Description |
| --- | --- |
| Quantum Annealing (QA) | D-Wave Advantage 6.4 (real quantum hardware) — uses quantum tunneling to explore the energy landscape and find low-energy (optimal) solutions. |
| QAOA | Gate-based quantum algorithm implemented in Qiskit — tested on simulators, limited to very small problem sizes. |
| Gurobi | Classical state-of-the-art solver (branch-and-bound, branch-and-cut) — used as the "gold standard" baseline. |

Problem Formulation

The grid is discretized into cells (e.g., 10×10). Binary variables s_x ∈ {0,1} indicate whether a turbine is placed in cell x. The objective maximizes power output with two constraints:

- Fixed number of turbines — exactly m turbines must be placed.

- Proximity constraint — turbines cannot be adjacent (for structural stability).

These are encoded as penalty terms in the QUBO objective:

text

Minimize: -Power(s) + λ₁(Σs_x - m)² + λ₂Σs_x·s_y

Where Power(s) is calculated using the Jensen wake model (Equation 4) and wind data (either uniform or real data from Los Angeles and Milan).

Methods

| Component | Details |
| --- | --- |
| Wake Model | Jensen model — calculates wind speed reduction behind each turbine. Linear superposition of power deficits (Equation 8) for computational efficiency. |
| Grid Size | Up to 10×10 cells (100 binary variables) — limited by D-Wave qubit connectivity. |
| Quantum Hardware | D-Wave Advantage 6.4 (real quantum annealer) — accessed via cloud. |
| Classical Baseline | Gurobi Optimizer — branch-and-cut with presolve routines. |
| QAOA | Qiskit framework — limited to very small problems due to circuit depth and qubit requirements. |
| Wind Data | Uniform distribution (Mosetti), realistic directional data, and real data from Los Angeles and Milan (Iowa Environmental Mesonet). |

Results

| Metric | Gurobi (Classical) | D-Wave Quantum Annealer | QAOA |
| --- | --- | --- | --- |
| Solution Quality | 100% (optimal/reference) | ~97% (at most 3% lower power output) | Poor — limited to tiny grids |
| Computational Speed | Slow (explores full search space) | 10x faster than Gurobi | Very slow (simulator) |
| Constraint Satisfaction | Always satisfied | Sometimes violates fixed number of turbines | Often violates constraints |
| Scalability | Poor (NP-hard) | Better for larger problems | Very poor (hardware limited) |

Key Findings

- Quantum annealing is fast but approximate — 10x faster than Gurobi, but produces solutions with at most 3% lower power output. This makes it suitable for rapid suboptimal approximations where speed matters more than perfection.

- QAOA is not ready — current quantum hardware restricts it to very small problems. It could not compete with either Gurobi or D-Wave.

- Trade-off is clear — speed vs. quality. Quantum wins on speed, classical wins on precision.

- Robustness — Quantum annealing performed particularly well in scenarios with higher solution space entropy (more complex configurations), allowing better exploration of alternative layouts.

- Statistical Validation — Wilcoxon signed-rank tests confirmed consistency between quantum annealer solutions and Gurobi solutions (p-values: ~10⁻¹⁵ for uniform wind, ~10⁻⁵ for real data).

- Hybrid Approaches Limited — Warm-starting Gurobi with quantum solutions and reverse annealing provided only marginal improvements.

Visual Results (Figure 4)

The paper shows candidate layouts for Gurobi, D-Wave, and hybrid solvers under uniform, realistic, and real wind conditions (Los Angeles and Milan). Blue grids indicate constraint satisfaction; red grids indicate violations.

Key Innovations

- First Comprehensive Comparison of quantum annealing, QAOA, and classical solvers on WFLO under both idealized and real wind conditions.

- QUBO Formulation for WFLO with realistic constraints (fixed turbines, proximity) — enabling direct implementation on quantum hardware.

- Real Wind Data Validation — tested on actual wind distributions from Los Angeles and Milan, not just toy problems.

- Speed-Quality Trade-off Quantified — clear demonstration that quantum annealing is 10x faster but 3% less accurate than Gurobi.

Limitations

- Small Problem Sizes — grid sizes limited to 10×10 (100 variables) due to D-Wave qubit connectivity and embedding overhead.

- Constraint Violations — quantum annealer sometimes violates the fixed number of turbines constraint (soft constraints).

- QAOA Impractical — requires too many qubits and too much circuit depth for current hardware.

- Noisy Hardware — D-Wave's noise characteristics not fully explored.

- Simplified Wake Model — uses Jensen model; more accurate models (Larsen) not tested.

- No Digital Twin — pure optimization, no simulation or virtual validation.